# A+UP Charter – Proactive Infrastructure Transformation Blueprint

Version 2.5: (Channel Spread, VoIP Jitter <30ms, mDNS Fallback, STP Alignment, Guest Throttle), November 10, 2025 Refinements

# Executive Summary

This document presents an updated, nearly deployment-ready blueprint for the A+UP Campus infrastructure. These enhancements include WiFi channel planning with 80MHz separation, VoIP fine tuning, implementing mDNS fallback, and throttling guest bandwidth to 25/10 Mbps. The solution features 16 UAP-AC-PRO wireless access points (three additional units to address coverage voids), headroom for up to 30 VoIP devices, dedicated 2.4GHz wireless capacity and mDNS for printers, and flexible camera deployment. The network is designed to achieve 92–96% wireless coverage for 130–140 Chromebooks.

The total upfront investment is ~$3,880, with a potential resale offset of approximately $1,200.

## Key Improvements (Project-Aligned)

- WiFi Optimization: Deployment of 16 UAP-AC-PRO units, including three new access points (two upstairs in rooms 207 and 205/hallway, and one downstairs in room 117). The 5GHz band is prioritized, utilizing non-DFS channels (36, 52, 100, 116, 132, 149) with 80MHz separation, or 40MHz as fallback. The 2.4GHz band is globally disabled except for a low-power segment (WAP2/4) reserved for printers. The RSSI threshold is set to -70 dBm, with steering at a minimum of -67 dBm. Initial testing will involve five Chromebooks, with adjustments based on feedback from a Week 1 survey. The expected outcome is that 92–96% of devices will experience signal strength better than -65 dBm, with less than 3% drop rate.

- Compliance & Resilience: The network will include deep packet inspection and filtering, support failover within five minutes, and utilize fiber for WAN demarcation.

- EdTech Support: The infrastructure is designed to handle Google service bursts of up to 100 Mbps per room, ensuring adequate density.

- Risk Mitigation: A Week 1 survey will identify any issues. Pre-shared key (PSK) and mDNS fallback (using Avahi/Bonjour if UniFi presents issues) are in place. Spanning Tree Protocol (STP) for camera networks is aligned for compatibility.

## Financial Summary

- Upfront Investment: $3,814, including $2,120 for hardware, $99 for CyberSecure services, and $805 for labor and diagnostics. An estimated $1,000 can be recouped through hardware resale, but only if school lets us decommission.

- Ongoing Costs: (if they are sold a service contract)$599 per month for Service-Only support, covering 10 hours of assistance.

- Three-Year Savings: Projected at $34,452, largely due to reduced overages and fewer reactive service calls.

| Metric | Value | Notes |
| --- | --- | --- |
| Upfront Cost | $3,814 | Offset ~73% via resale |
| Monthly Recurring | $599 | Service-Only stabilization |
| Client 3-Year Savings | $34,452 | From overages and reactive fixes |
| Estimated Client ROI | Immediate | Overage bleed stops month 1 |

## Core Infrastructure

- Gateway & Controller: UDM Pro Max with 10G capability, integrated IDS/IPS, UniFi OS, and fiber WAN via UF-SM-1G-S SFP (verified 1000BASE-LX 1310nm/10km).

- Distribution: USW-Pro-Max-48-PoE switch, offering a 720W PoE budget and 10G uplink. Thirty-nine out of forty-eight ports are allocated, with TP-Link switches flexed for camera connections.

- Wireless: Sixteen UAP-AC-PRO access points (including three additional units for coverage), with a Cloud Key Gen2 retained for management.

- Endpoints: Approximately eight Yealink SIP-T43U phones on VLAN50, twelve to fifteen Verkada cameras on VLAN60 (model-specific PoE), and ad-hoc wireless printers on VLAN20.

- Power: APC SMX2000LVNC UPS provides 8–12 minutes of runtime, maintaining PoE usage below 40% of baseline capacity.

# Network Architecture

## VLAN: Design & Capacity

| VLAN | Purpose | Subnet | Usable Hosts | DHCP Lease | QoS Profile |
| --- | --- | --- | --- | --- | --- |
| 10 | Students Chromebooks | 10.10.0.0/23 | 510 | 4 hours | Standard (Content Filtering) |
| 20 | Staff Devices/Printers | 10.20.0.0/24 | 254 | 24 hours | High Priority (mDNS enabled) |
| 30 | Guest WiFi | 10.30.0.0/24 | 254 | 2 hours | Throttled (25Mbps down/10Mbps up, full isolation) |
| 50 | VoIP (Yealink) | 10.50.0.0/27 | 30 | Infinite | EF (Expedited Forwarding) |
| 60 | Cameras (Verkada) | 10.60.0.0/26 | 62 | Infinite | AF41 (Assured Forwarding) |
| 99 | Management | 10.99.0.0/28 | 14 | Infinite | CS6 (Network Control) |

Note: VLAN50 /27: 30 usable; DHCP 10.50.0.2–30. VLAN10: 4hr for transients. Printers: Ad-hoc 2.4GHz + mDNS reflector (UDM > Networks > Multicast DNS; fallback Avahi/Bonjour if AirPrint unreliable). MTU: 1500 consistent.

## Firewall Rules: Security Posture

| VLAN | Rule # | Source | Destination | Ports/Protocol | Action | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 30 Guest | 1 | 10.30.0.0/24 | 10.30.0.1 | 53,67,68 (DNS/DHCP) | Allow | No HTTP to gateway. |
| 30 Guest | 2 | 10.30.0.0/24 | 10.0.0.0/8 | All | Block | Internal isolation. |
| 30 Guest | 3 | 10.30.0.0/24 | 0.0.0.0/0 | All | Allow | Internet only. |
| 30 Guest | 4 | SSID "A+UP-Guest" | Internal | N/A | Client Isolation | No inter-guest. |
| 50 VoIP | 1 | 10.50.0.0/27 | Spectrum SIP | 5060-5061,8000-8800 | Allow | SIP/RTP bidirectional. |
| 50 VoIP | 2 | 10.50.0.0/27 | 10.20.0.0/24 | All | Allow | Staff dialing. |
| 50 VoIP | 3 | 10.50.0.0/27 | 10.10.0.0/23,10.30.0.0/24 | All | Block | No student/guest. |
| 60 Cams | 1 | 10.60.0.0/26 | Verkada Cloud | HTTPS 443 | Allow | Cloud outbound. |
| 60 Cams | 2 | 10.99.0.0/28 | 10.60.0.0/26 | 554 (RTSP) | Allow | Mgmt pull. |
| 60 Cams | 3 | 10.60.0.0/26 | Other VLANs | All | Block | Segmentation. |
| 99 Mgmt | Add | 10.99.0.0/28 | 10.60.0.250 | 443 (HTTPS) | Allow | TP-Link GUI (if Cam Island active). |

## PoE Power Budget & Future-Proofing

| Scenario | Load | Utilization | Headroom | Notes |
| --- | --- | --- | --- | --- |
| Current (16 APs, 8 Phones) | 290W | 40% | 430W (60%) | 144W APs + 26W phones + 120W cams; model-specific (af/at). |
| Future (WiFi 6E) | 450W | 63% | 270W (37%) | U6 @16W ea. + spikes. |
| Emergency Buffer | - | - | 150W | Firmware/heat. |

## Physical Topology Overview

The network topology has been confirmed with 39 out of 48 ports utilized, ensuring no overlap in assignments. The fiber handoff from Comcast uses an LC connector to the UDM SFP+ port (UF-SM-1G-S, $15; 1310nm validated). A 10G LACP uplink connects to the USW switch, with the following dedicated port allocations:

- Access Points (APs): Ports 1–16 (16 ports)

- VoIP Devices: Ports 17–24 (8 ports)

- Cameras: Ports 25–39 (15 ports)

- Spares: Ports 40–48 (9 open ports, including UPS on Port 48)

AP refinements include the deployment of 10 APs upstairs (adding channels 36, 52, and 100) and 6 APs downstairs (adding channels 116 and 132, with WAP13 relocated to Room 107). The 2.4GHz band is specifically carved for WAP2 and WAP4.

A contingency plan for the camera island involves trunking USW Port 40 to TP-Link Port 24. Ports 1–15 on TP-Link are assigned to ACCESS VLAN60 (PVID60) with PoE automatically or forcibly set depending on the model. Management IP is set to 10.60.0.250, with RSTP alignment using the 802.1w protocol. The line of demarcation (LoD) runs from Comcast to the demarcation point, and internal patching is handled from there. Optical power measurements should exceed -15 dBm.

## Technical Enhancements

- Chromebook Optimization: Enable 802.11k/v/r protocols; set -67 dBm as the steering minimum (testing with 5 units for sticky client behavior and adjusting the threshold to -70 dBm based on survey results). Implement DPI for Google traffic and maintain handoffs under 5 seconds.

- VoIP: Use the G.722 codec with DSCP EF tagging, targeting jitter below 30ms and latency under 150ms. Disable ALG and ensure VLAN50 can scale to support 30 devices.

- Verkada Cameras: Configure for 100 Kbps idle and 3 Mbps live streams (with potential bursts up to 45 Mbps). Set AF41 for priority and force PoE at the device for models CD52 and CD62.

- Printers: Establish an ad-hoc VLAN20, carve the 2.4GHz band, and enable mDNS (UDM > Networks > Multicast DNS; fallback to Avahi/Bonjour for AirPrint compatibility). Inventory all printer models for 2.4GHz compatibility.

- Guest Network (VLAN30): Implement a captive portal, bandwidth limits of 25 Mbps down/10 Mbps up, and maintain 90-day access logs.

- Monitoring: Utilize UniFi Insights for monitoring PoE, VLAN, and WAN status; enable SNMP monitoring for UPS devices and track RSSI. The MTU is set to 1500.

## Pre-Lab Go/No-Go Checklist

- Blocking Tasks (By Nov 14): Validate Comcast handoff specifications (including optical and connector details); conduct Chrome 802.11k/v testing on 5 units; check SFP modules; prepare the lab bench; establish rollback procedures; update the Terri matrix; deploy 3 APs; confirm TP-Link stock and verify RSTP functionality.

- Tasks Before Nov 17: Distribute PSK credentials; conduct mDNS lab tests (including AirPrint fallback); review Verkada camera models for PoE preferences; complete printer inventory for 2.4GHz and mDNS compatibility.

## Deployment Timeline

| Phase | Dates | Key Activities | Owner | Success Metrics |
| --- | --- | --- | --- | --- |
| Pre-Deployment Align | Nov 12–14 | Matrix; printer probe (inventory); Cat6; Zoom (RADIUS/PSK/CB tests/TP-Link RSTP). | Joint | 98% viability. |
| Lab Forge | Nov 17–23 | VLAN/DPI; iPerf; 16× AP tuning (ch spread, 2.4GHz carve); RADIUS/PSK; failover/Verkada (45 Mbps, model PoE); mDNS (fallback test); Cam Island trunk/STP. | Travis | 20-client; <3% drops; Terri videos. |
| Backbone Pilot | Nov 25 | Rack/WAN/UDM/USW; legacy decom; VLAN (/27); verifs; Terri call. | Travis | PoE <40%; 99% uptime. |
| AP Swarm + RADIUS | Nov 26 | 16× adoption (adds first); SSID/RADIUS/PSK; roaming (30 CBs); printer carve; Terri push. | Travis/Terri | 92–96% coverage; <5s roaming. |
| Integrated Polish | Nov 28 | Tests (jitter <30ms/VLAN60/RADIUS/mDNS); walk (AP/printer); draft. | Joint | <30ms jitter; cross-VLAN printing. |
| Buffer Tune | Dec 1 | Tweaks (DFS/PoE/ch overlap); Week 1 call; ROI. | Travis | Voids +12–15 dB. |
| Handoff Horizon | Dec 2 | Async (RSSI/mDNS videos); support; Terri check-in (buffer Dec 3 if delay). | Terri | Live; Dec 4 queue. |

## Rollback Checkpoints

#### Incident Response Phases and Actions

| Phase | Trigger | Action |
| --- | --- | --- |
| Lab Forge | WAN throughput drops below 800 Mbps or optical signal strength falls below -20 dBm | Initiate a JSON configuration revert and document the issue with the ISP. |
| Backbone | Reprovisioning takes longer than 10 minutes or there is a network flap | Insert Juniper equipment and establish a loopback to resolve. |
| AP Swarm | More than two adoption failures occur or roaming latency exceeds 10 seconds | Update firmware and defer adding new devices until stability is confirmed. |

## Post-Deployment Checklist

### Network Configuration

Ensure that all network rules and VLAN settings are active, including the /27 subnet configuration and support for mDNS. Verify successful adoption of all required devices: 16 access points, 8 phones, and 15 cameras.

### Performance Validation

- Wireless network throughput exceeds 500 Mbps.

- Jitter is maintained below 30 ms.

- Latency does not exceed 150 ms.

- WAN throughput is greater than 900 Mbps.

- Power over Ethernet (PoE) usage is at 40%.

- Burst traffic rates reach up to 45 Mbps.

### Project Handover Documentation

Prepare a comprehensive handover PDF that includes the following:

- Network diagram detailing the 16 access points, VLAN assignments, levels of detail, and camera flexibility.

- Runbook outlining operational procedures.

- Credentials, including pre-shared key (PSK) information.

- Network tree structure.

- Validation results for controller board and mDNS functionality.

- System logs and scheduling details.

### Training Plan

Provide a two-hour training session covering SSID configuration, device communication, VLAN assignments, and mDNS usage. For the first week, grant read-only access to participants.

## Risk & Mitigation Table

| Gap | Risk lvl | Mitigation | Owner |
| --- | --- | --- | --- |
| Terri Slip | High | Nov 13 matrix/videos; Dec defer. | Joint |
| Lab Creep | Low | 3–4h cap; RADIUS/AP/mDNS prio. | Travis |
| Cat6 | Low | Blueprints; injectors. | Joint |
| Verkada | Low | VM (Nov 22); app (Dec 2); 45 Mbps QoS/model PoE. | Joint |
| Printers | Medium | Inventory (2.4GHz/mDNS compat); carve + fallback. | Joint |
| RADIUS | Medium | Google stress (Nov 21); PSK doc. | Joint |
| Coverage Voids | Medium | Week 1 survey; -70 dBm. | Travis |
| Chrome Sticky | Low | Pre-lab test 5 units; -67 dBm adjust. | Travis |
| Comcast Delay | Medium | Buffer Dec 3; specs Nov 14. | Travis |
| Port Squeeze | Low | Cam Island (TP-Link trunk/RSTP align); ACCESS mode. | Travis |
| Channel Overlap | Low | 80MHz spread (36/52/100/116/132/149); 40MHz fallback. | Travis |
| mDNS Quirks | Low | UniFi primary; Avahi/Bonjour fallback. | Travis |

## Costs & ROI (Client-Focused)

Upfront Breakdown

| Category | Item | Qty | Price | Total |
| --- | --- | --- | --- | --- |
| Hardware | UDM Pro Max | 1 | $599 | $599 |
| Hardware | USW-Pro-Max-48-PoE | 1 | $1,299 | $1,299 |
| Hardware | SFP Module | 1 | $15 | $15 |
| Hardware | UAP-AC-PRO Adds | 3 | $149 | $447 |
| Licenses | CyberSecure | 1 | $99 | $99 |
| Labor | Diagnostics & Setup | 1 | $700 | $700 |
| Total | - | - | - | $3160 |

## Handover Documentation

To ensure a smooth transition and ongoing operational success, the following enhanced handover documentation package is provided:

### Documentation Format

- All documents will be delivered in both PDF and Word formats for compatibility and ease of use.

### Network Diagrams

- Network topology diagrams will illustrate both the pre- and post-implementation topologies.

- Diagrams will include detailed mappings of VLANs, Levels of Detail (LoD), SFP connections, 2.4GHz radio layouts, and Camera Flex (STP topology).

### Credential Vault

- All sensitive credentials and pre-shared keys (PSKs) will be stored in an encrypted vault to ensure security and controlled access.

### Runbook Guides

- Concise, five-minute operational guides will be provided for key systems, including Access Points (AP), VoIP, mDNS, PSK, WAN, and the Camera Island configuration.

### Network Tree and Flowchart

- A Tier 1 flowchart will outline the primary support and troubleshooting path, incorporating sticky sessions and RADIUS authentication procedures.

### Validation Procedures

- Lab validation steps will cover core checks, including circuit breakers (CB), mDNS operation, sustained throughput at 45 Mbps, and Spanning Tree Protocol (STP) verification.

### Logging and Scheduling

- Documentation will include schedules for ongoing updates as well as checklists for firmware and mDNS verification and maintenance.

### Training Program

- A two-hour training session will be provided, covering SSID management, Talk application usage, VLANs, and mDNS. The training will be phased and initially offered in a read-only format for gradual onboarding.

## Appendix A: Advanced Configuration

### LACP Trunk Configuration

To configure the Link Aggregation Control Protocol (LACP) trunk on the UniFi Dream Machine (UDM) via SSH, follow these steps:

- Access the UDM through SSH.

- Enter configuration mode by typing: configure.

- Set the bonding interface policy with: set interfaces bonding bond0 hash-policy layer2+3.

- Commit the changes with: commit.

- Save the configuration by entering: save.

### WiFi Tuning Recommendations

For optimal wireless performance, apply the following WiFi tuning adjustments:

- Channel spread: Use channels 36, 52, 100, 116, 132, and 149 (each at 80 MHz).

- Increase transmit power by +7 dB.

- For 2.4GHz, implement a frequency carve by navigating to Radios > Legacy in the interface.

### SFP Module Selection

For Small Form-factor Pluggable (SFP) modules, use the UF-SM-1G-S model (1000BASE-LX). As an alternative, the UF-SM-10G module may also be utilized.

### mDNS Configuration

Enable Multicast DNS (mDNS) in the UDM by navigating to Networks and toggling the mDNS option. If necessary, fallback to Avahi or Bonjour for mDNS services.

### Camera Island Network Settings

For Camera Island configuration:

- On the USW switch, configure Port 40 for Native VLAN 1, Tagged VLAN 60, and enable all VLANs with Spanning Tree Protocol (RSTP 802.1w).

- On the TP-Link switch, assign IP address 10.60.0.250 to Management VLAN 60.

- Set Port 24 as a trunk port with PVID 1 and Tagged VLAN 60.

- Configure Ports 1–15 as access ports with PVID 60.

- Enable RSTP and ensure PoE is supplied for CD52 devices.

### PoE Verification

Monitor Power over Ethernet (PoE) using Insights. Confirm that the total current does not exceed 290W.

### Lab Results and Testing

After November 23, conduct lab testing using iPerf and Wireshark. Focus on mDNS, PSK, and STP performance metrics.

### Rollback Procedures

If a rollback is required, follow these timing guidelines:

- WAN rollback: Allow 2 minutes.

- PoE rollback: Allow 5 minutes.

- VLAN rollback: Allow 10 minutes.

## Vendor Contacts

- UniFi: For assistance with UniFi products, contact support via email at support@ui.com or by calling 1-877-478-2439.

- Spectrum SIP: For Spectrum SIP-related support, reach out by phone at 1-855-707-7328.

- Verkada: To obtain support for Verkada devices, email support@verkada.com.

- APC/UPS: For APC/UPS matters, call 1-888-545-0874.

- Comcast: For Comcast support, dial 1-800-COMCAST.

## Final Notes

Refer to the vendor contact information above for any troubleshooting, support requests, or questions related to device setup and maintenance. Ensure all relevant parties are aware of these contacts for efficient communication during the deployment and testing process.