# Comprehensive WiFi Assessment Summary V2.0: A+UP Charter School

Technical Analysis for Future Network Engineering Reference – November 2025

# 1. Building Overview

Facility: 20,205 sq ft (approx. 10,000 sq ft per floor), renovated with Cat6 in 2021, featuring a central network closet.

Location: Urban Houston (highway edge, low ambient noise; 180–230 SSIDs detected per scan).

Existing Infrastructure: 13 × UAP-AC-PRO (WiFi 5, ceiling mounts; 7 upstairs, 6 downstairs), Cloud Key Gen2.

Device Density: 110–120 Chromebooks upstairs; 5-device cart downstairs.

Key Network Strains:

- 30–50% packet drops due to coverage gaps and roaming (5–10s disruptions)

- 2.4GHz: 70–80% channel saturation, 40–60% collision rates

- 5GHz: 30–40% airtime loss, 40–62 neighboring SSIDs per scan

Note: EdTech usage spikes (Google Meet, 100 Mbps/room) exacerbate existing coverage voids.

Survey Methodology: NetSpot (10+ data points per floor, SSID-tied screenshots). Note: Survey bias toward core areas; unsurveyed voids may result in 10–15% overstatement.

RF Model: RSSI model, n=3.5/4 (three-wall average, 5GHz, 20 dBm Tx, 100x80 ft grid): Mean RSSI -62 dBm upstairs / -57 dBm downstairs. Post-tuning: +6 dB improvement (75–82% → 90–95% coverage above -65 dBm).

# 2. Current Deployment & Proposed Hardware

Core Network: UDM Pro Max / USW-48-PoE ($1,673 + $260 optional adds).

Lab Pre-Tuning: Channel and power optimization; globally disable 2.4GHz; enable 802.11k/v/r; set RSSI threshold to -70 dBm.

Top-Line Coverage:

- Upstairs: 82% (density constraints); projected 90% with two additional APs and rebalance

- Downstairs: 85% (lean load); projected 93% with AP relocation

# 3. Second Floor – High-Density Core (~110 Chromebooks)

## 3.1. Load & Traffic Patterns

- Classroom bursts: 15–30 devices per room, 100–200 Mbps/class (2018–2022 WiFi 5 fleet)

- Library (Rm207): 20–30 simultaneous users

- Hallways: 50+ concurrent roaming devices

## 3.2. Access Point Deployment Snapshot (7 APs)

| AP ID | Location | Role | Issues (RSSI Est/Sim) |
| --- | --- | --- | --- |
| WAP1 | NW (Rm201 adj, 30° tilt) | Classrooms 201 | Fair (-65 dBm edges); tilt causes ~10% coverage loss. |
| WAP2 | Office (Rm208) | Admin | Redundant with WAP4 (-50 dBm); low-density inefficiency. |
| WAP3 | Rm201 | Classroom | Strong (-55 dBm); solid density support. |
| WAP4 | Office/Rm208 | Admin | Redundant with WAP2; signal bleed into Rm207 (-70 dBm). |
| WAP5 | Rm202 | Classroom | Strong (-58 dBm); 20+ Chromebooks supported well. |
| WAP6 | Rm206 | Classrooms 206/edge | Coverage stretched to Rms 205/207 (-68 dBm); lag risk present. |
| WAP7 | Rm204 | Classroom east | Strong (-56 dBm); no current issues. |

Coverage Baseline: 75% above -65 dBm (core 85%, edges 55%); mean -62 dBm; 30% loss attributed to channel overlap and congestion.

## 3.3. Coverage Gaps & Overages

- Rm207 Library (P1): No dedicated AP; -70 to -80 dBm (60 ft from WAP5/6, 3+ walls, unsurveyed); 20–30 Chromebooks, 40–50% drop rate.

- Recommendation: Add WAP8 (ceiling center, channel 165 high, +15 dB to -55 dBm; aim for 100% reliability).

- Rm205 (P2): -65 to -75 dBm (30–40 ft from WAP6, unsurveyed); lag for flex/storage cart.

- Recommendation: Add WAP9 (door edge, channel 36 medium, +12 dB to -53 dBm) or relocate WAP9 from 1st-floor overlap.

- Stairwells (P3): -70 to -82 dBm vertical; 5–10s drop risk.

- Recommendation: Enable 802.11r/k/v and set RSSI threshold to -70 dBm; perform walk survey in Week 1.

- Office (WAP2/4): Redundant -50 dBm (20 ft, 20% over-provision).

- Recommendation: Lower WAP2 power, repurpose if feasible.

Survey Bias Note: Core-focused surveys may overstate coverage by 10%; unsurveyed voids present a 15% risk. Week 1 re-survey recommended.

## 3.4. RF & Channel Deployment

- 2.4GHz: 70–80% saturation (channels 1/6/11, 12–25 neighboring APs). Disable globally; configure WAP2 at low power for printers only.

- 5GHz: 30–40% airtime loss; channels 36/165 clean (4–8 neighbors), avoid 112/149 (10–15 neighbors). DFS channels (52/100) are acceptable (2–4 neighbors, 1–2% radar events).

- Lab Tuning: Channels 36/165/157, 80MHz width. Simulated improvement: mean RSSI +7 dB (75% → 92% above -65 dBm); voids improve by +12–15 dB.

# 4. First Floor – Low-Density Secondary (~5 Chromebooks, Cart-Based)

## 4.1. Load & Traffic Patterns

- Device cart roams, generating 20–50 Mbps bursts

- Primary use: admin and storage; no high-density classrooms

## 4.2. Access Point Deployment Snapshot (6 APs)

| AP ID | Location | Role | Issues (RSSI Est/Sim) |
| --- | --- | --- | --- |
| WAP8 | SW (Rms102–106) | Classrooms | Strong (-55 dBm); cart coverage solid. |
| WAP9 | Four Rooms | Classrooms | Solid (-58 dBm); minor SW coverage stretch. |
| WAP10 | Central (Common 100) | Hall/common | Excellent (-43 to -50 dBm); well-positioned. |
| WAP11 | NE (Rm111) | Classrooms 109–111/BR | Fair (-61 dBm); BR at -65 dBm is marginal. |
| WAP12 | SE (Elevator, Rms118–120) | Offices/elevator | Strong (-42 to -57 dBm); vertical coverage is optimal. |
| WAP13 | Storage/Mech | Low-traffic | Over-provisioned (-58 to -62 dBm); redundant with WAP10. |

Coverage Baseline: 78% above -65 dBm (core 85%, edges 60%); mean -57 dBm (upstairs +5 dB). 25% coverage loss from coverage stretches.

## 4.3. Coverage Gaps & Overages

- Rm107 NW Corner (P1): -68 to -75 dBm (40–60 ft from WAP9/13, 3 walls, unsurveyed); cart/library faces 30% drop risk.

- Recommendation: Relocate WAP13 (channel 36 high, +15 dB to -53 dBm; 85% reliability expected).

- Rm117 NE Corner (P2): -65 to -72 dBm (40 ft from WAP11, obstructed, unsurveyed); lag risk for secondary use.

- Recommendation: Increase WAP11 power (test for potential WAP12 channel clash) or relocate WAP9 (channel 149 medium, +12 dB).

- Rm10 Central (P3): Notation may be obsolete; -43 to -58 dBm (WAP10).

- Recommendation: Confirm status as of Nov 13 ("Rm10 complaints?"); disregard if storage.

- Stairwells: Horizontal coverage fair (-55 to -65 dBm); vertical risk of 5s drops.

- Recommendation: Enable 802.11r; conduct walk survey in Week 1.

- Central (WAP10/13): Redundant -50 dBm (20–30 ft, 15% over-provision).

- Recommendation: Relocate WAP13 to Rm107; set to low power prior to move.

Survey Bias Note: 80% of survey points are near APs; Rms 107/117 unsurveyed (10% coverage overstatement risk). Cart-based probing ("In 107/117?") recommended for pattern analysis.

## 4.4. RF & Channel Deployment

- 2.4GHz: 70–80% saturation (channels 1/6/11, 12–20 neighboring APs). Disable; carve out WAP8 for printers if necessary.

- 5GHz: 20–30% airtime loss; channels 36/165 clean (4–8 neighbors), avoid 112/149 (10–12 neighbors).

- Lab Tuning: Channels 36/149/157, 80MHz width. Simulated improvement: mean RSSI +6 dB (78% → 94% coverage above -65 dBm); voids improve by +12–15 dB post-relocation.