# Yealink T43U VoIP Migration & Configuration Manual

From Spectrum Box to Direct SIP on UniFi USW-Pro-Max-48-PoE

# Introduction

This manual provides a concise, lab-ready guide for migrating 8–12 Yealink T43U phones from a Spectrum-provided ATA/proxy (the "Spectrum box") to direct SIP registration on VLAN 50, utilizing UniFi UDM DHCP and USW-Pro-Max-48-PoE (ports 14–25). It preserves existing extensions, voicemail, and paging via the Spectrum cloud. The procedure is optimized for technical clarity, minimal downtime, and repeatable success.

- Estimated Total Time: 4–6 hours (Lab: 2 hours; Production: 2–4 hours)

- Risks: NAT traversal issues (ALG must be off), DHCP reservation glitches, RTP/codec mismatches

- Lab Hero: Borrowed T43U for pre-production testing

- Decommission: Remove Spectrum box only after successful lab validation

## Technical Highlights & Change Log

- RTP port range initially wide (10000–65535); narrow after Wireshark analysis (Post-Migration)

- STUN tested only if NAT/one-way audio is present; static IPs preferred

- Multicast paging limited to VLAN 50 unless IGMP routing is configured

- G.722 codec prioritized (confirm with Spectrum); fallback to PCMU/PCMA

- Wireshark RTP capture mandated in lab validation

- Sections on limitations and optimization included for ongoing improvement

# Phase 1: Pre-Migration Preparation (2 Hours)

## Step 1.1: Extract Spectrum SIP Credentials

Objective: Obtain all required SIP registration details for direct phone provisioning.

- Method A: Spectrum Portal (Preferred)

- Log in to Spectrum Portal: https://business.spectrum.com/voice or https://voice.spectrum.com

- Navigate: Settings → SIP Trunks → Account Details

- Collect per line:

- SIP Username (e.g., 7135551234@sip.spectrum.net)

- SIP Password (12–16 chars)

- SIP Server/Proxy

- SIP Port (5060 UDP)

- Outbound Proxy (if required)

- STUN Server (ask Spectrum; test without first)

- Export CSV if available (Settings → Extensions)

- Confirm G.722 support with Spectrum

- Method B: Spectrum Box UI (Fallback)

- Access via IP: http://192.168.0.1 (see label)

- Login: admin / [password from label]

- Navigate: Voice → Line 1–4 → SIP Settings

- Screenshot all credentials and server fields

- Method C: From T43U Phone (Last Resort)

- Menu → Settings → Advanced (pass: admin) → Accounts → SIP Server

- Note: Passwords are obscured; portal preferred

## Step 1.2: Inventory Phone Configuration

Objective: Document all phones and their current configurations for accurate reprovisioning.

- Create a "VoIP Hero List" spreadsheet (Google Sheets/Excel):

| Phone # | MAC Address | Current IP | Extension | Voicemail PIN | Paging Group | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 80:5e:c0:xx:xx:01 | 10.x.x.101 | 101 | 1234 | All Staff | Front desk |
| 2 | 80:5e:c0:xx:xx:02 | 10.x.x.102 | 102 | 5678 | Teachers | Room 201 |

- Gather data per phone: Menu → Status → Network → IP; Status → Phone → MAC

- Export from Spectrum Portal: Settings → Extensions

- Prioritize 8 active phones for lab; document up to 12

## Step 1.3: Lab-Test Direct SIP Registration

Objective: Validate direct SIP registration and core call features in a controlled environment.

- Connect T43U to laptop via Ethernet (laptop acts as DHCP server, gateway 10.50.0.1/28; range 10.50.0.2–10.50.0.14)

- Access T43U Web UI (http://10.50.0.10 or via DHCP lease); login as admin

- Configure Account 1 with extracted SIP credentials (see Step 1.1)

- Network: NAT Traversal set to STUN (test without first; enable if one-way audio)

- Codec Priority: G.722 (if supported), fallback to PCMU/PCMA

- Multicast Listening: 224.0.1.75, Label "All Staff", Codec G.711, Priority 1

- Run Wireshark on laptop, filter for RTP during a test call; document RTP port range used

- Test registration, extension-to-extension calls, voicemail access, and paging group

- Check RTP jitter (<150ms under simulated load)

- If any test fails, halt migration and escalate to Spectrum; otherwise, proceed to production

# Phase 2: Switch & Network Configuration (1 Hour)

## Step 2.1: Create Voice Port Profile

- In UniFi Controller/UDM GUI: Settings → Profiles → Switch Port Profile → Create New

- Profile Name: VoIP-Phones-VLAN50

- VLAN: Native VLAN 50 (access port, untagged)

- PoE: Mode Auto (802.3af/at); disable PoE+ (T43U needs only 7W)

- Advanced:

- Port Isolation: Disabled

- Storm Control: Enabled (10% for paging)

- Spanning Tree: Enabled, Edge Port

- Link Aggregation: Disabled

- Priority 5 (DSCP EF/46)

- LLDP-MED: Enabled

- Apply to USW-Pro-Max-48-PoE ports 14–25

## Step 2.2: Verify LLDP-MED

- Enable LLDP and LLDP-MED globally; set Voice VLAN 50, Priority 5

- Verify in UniFi Controller: Devices → USW-Pro-Max-48-PoE → Ports → Port 14 → Insights → LLDP Tab

- Expected: T43U detected, VLAN 50, Priority 5

- CLI Alternative: show lldp neighbors (see port, MAC, VLAN)

## Step 2.3: VLAN 50 Network Settings

- Settings → Networks → Create New

- Name: VoIP-Phones

- VLAN ID: 50

- Gateway/Subnet: 10.50.0.1/28

- DHCP: Server; Range 10.50.0.2–10.50.0.14; Lease Time: Infinite

- DNS: 10.50.0.1 (or 8.8.8.8)

- Advanced: mDNS Disabled, IGMP Snooping Enabled, IPv6 Disabled

## Step 2.4: DHCP Reservations

- Settings → Networks → VoIP-Phones → DHCP → Fixed IP

- Assign each T43U's MAC to a fixed IP (use spreadsheet from Step 1.2)

- Verify via UDM Clients (filter by VLAN 50)

# Phase 3: Firewall & QoS Configuration (1 Hour)

## Step 3.1: Firewall Rules (VLAN 50 Traffic)

| Rule Name | Action | Source | Destination | Ports/Protocol | Notes |
| --- | --- | --- | --- | --- | --- |
| VLAN50-to-Spectrum-SIP | Accept | VLAN 50 | Internet/Spectrum | TCP/UDP 5060–5061, 10000–65535 | SIP/RTP; log; narrow after Week 2 |
| VLAN50-to-VLAN20 | Accept | VLAN 50 | VLAN 20 (Staff) | All | Staff dialing |
| VLAN50-Block-Students-Guests | Drop | VLAN 50 | VLAN 10, 30 | All | No student/guest calls; log |
| VLAN50-Multicast-Paging | Accept | VLAN 50 | 224.0.1.75 | UDP 5060 | Paging multicast; VLAN 50 only |

## Step 3.2: Disable SIP ALG

- On UDM CLI:

- ssh admin@192.168.1.1

- configure

- set system conntrack modules sip disable

- commit; save

- exit

- # Verify: show system conntrack modules (should show sip: disabled)

## Step 3.3: QoS Configuration

- Settings → Internet → WAN Networks → Primary → Smart Queues: Enable (Download/Upload: 420 Mbps)

- Create Traffic Rule:

- Name: VoIP-Priority

- Protocol: UDP

- Source: VLAN 50

- Destination: Internet

- Ports: 5060–5061, 10000–65535

- DSCP: EF (46)

- Rate Limit: None (unlimited for voice)

# Phase 4: Migration Execution (2–4 Hours, Off-Hours)

## Step 4.1: Bulk Provisioning via TFTP (Recommended for 12 Phones)

- Install TFTP server (e.g., tftpd64 for Windows) on UDM or laptop

- Create global template (y000000000000.cfg) with standardized SIP and network settings

- Create MAC-specific config files (e.g., 805ec0abcd01.cfg) with unique credentials

- Set DHCP Option 66 to tftp://10.50.0.1

- On phones: Menu → Settings → Auto Provision → Server URL (or via DHCP auto-provision)

## Step 4.2: Manual Provisioning (For <8 Phones)

- Connect T43U to USW ports 14–25 (VLAN 50, PoE auto)

- Access Web UI (http://10.50.0.x) and configure with SIP credentials from Step 1.1

- Set Network → VLAN ID = 50

- Enable STUN only if required (test for one-way audio)

- Set Codec: Preferred G.722, secondary PCMU/PCMA

- Configure Multicast: 224.0.1.75, Label "All Staff", Codec G.711 (0)

- Save and reboot phone

## Step 4.3: Testing & Validation

- Test extension-to-extension calls

- Test voicemail access (*98)

- Test paging group broadcast

- Under simulated load, confirm RTP jitter <150ms (Wireshark: "sip || rtp || rtcp")

## Step 4.4: Decommission Spectrum Box

- If all tests pass: Power off and store Spectrum box as fallback (optionally resell)

- If failures: Retain box and troubleshoot; escalate to Spectrum support as needed

# Testing & Validation Checklist

- [ ] SIP registration: "Registered" displayed

- [ ] Extension call: Connects, no one-way audio

- [ ] Voicemail retrieval: <10s

- [ ] Paging group broadcast: No jitter (<150ms)

- [ ] DHCP reservation: IP consistent after reboot

- [ ] LLDP-MED: Phone auto-discovers VLAN 50

# Troubleshooting

- Registration Fails: Verify SIP credentials, disable SIP ALG, capture with Wireshark

- One-Way Audio: NAT issue—test STUN/outbound proxy; confirm settings

- No PoE: Confirm port profile, check USW PoE status

- Paging Drops: Ensure IGMP snooping/storm control enabled, paging on VLAN 50 only

# Known Limitations

- RTP Port Range: Wide by default (10000–65535); narrow after Week 2 Wireshark analysis

- Multicast Paging: VLAN 50 only; cross-VLAN requires IGMP routing

- Codec Support: G.722 prioritized; confirm support with Spectrum before deployment

# Post-Migration Optimization (Week 2)

- Narrow RTP Firewall Rules:

- Capture 24 hours of RTP traffic (Wireshark), identify active port range

- Update firewall and QoS rules accordingly

- Verify Codec Usage:

- Check phone status (Menu → Status → Codec)

- If G.722 is not active, reprioritize to PCMU

- Optimize Multicast Paging:

- Test paging to multiple groups

- If cross-VLAN paging needed, enable IGMP routing on UDM

This manual is your technical playbook—test thoroughly in the lab, then execute the migration with confidence. For 12 phones, choose bulk (TFTP) for speed or manual for granular control.